# XTEND 4.0

REPEATER 136-174MHz

Ready-to-use, high gain, digital filter (option) & rugged repeater Multi modulations operation with TruPWR™ (TETRA/INPT/DMR/ANALOG) Cost-effective remote unit (+17/+30dBm)

RFFO (optical fiber) or RFRF operation

# Key features

Ultra-fast 100µs squelch / timeslot ALC based to avoid near BS desensitisation And to allow a large number of MS connections along the RU

In-house WDM RFoG system up to 16 RUs and distance of 35km  Remote unit with low and high-power cost-effective solution  Secured Talk2M© monitoring cloud from EWON© (option)

Low time equipment deployment thanks to plug & play system

Full compatible with MDAS© product range

# Benefits

Reliability and easy maintenance with no RU with FAN  Low monitoring development cost with the Talk2M©

Unconditionally stable and usable with existing optical fiber  Monitoring over optical fiber

# OPTICAL FIBER DAS & RFRF REPEATER

Poor or inexistent PMR coverage is a problem for public safety infrastructure (building, tunnel…).

We are providing a full range of 136-174 MHz indoor repeater, for all kind of application.

  - OPTICAL FIBER DAS (DISTRIBUTED ANTENNAS SYSTEM)

Figure 1 RFFO REPEATER

Our DAS is using an in-house master unit (MU), with the following advantages:

    - Network safe with no noise guaranteed

    - Uplink dynamic ALC & ultra-fast 100µs squelch (uplink muting with no MS)

    - Digital filter (option)

    - In-house monitoring for critical network with MOR (minimum operating requirements)

    - Plug & play and ultra-secured monitoring platform with Talk2M©

Our DAS is using an in-house analog optical master unit (OMU), with the following advantages:

    - Single mode optical fiber

    - Long distance up to 35KM achievable

    - Up to 16 remote unit with only 6x 1U OMU

Our DAS is using an in-house remote unit (RU), with the following advantages:

    - Cost effective solution

    - In-house monitoring for critical network with MOR (minimum operating requirements)

    - Optional leaky feeder monitoring, bias voltage for in-line BDA and custom request

    - Local I/O (option)

  - RFRF REPEATER

Figure 2 RFRF REPEATER

Our RFRF repeater provides ready-to-use, high gain, digital filter (option) and high power (option) in a compact box:

    - Plug & play with automatic RF setup

    - Network safe with no noise guaranteed

    - Up to 90dB of gain

    - Uplink dynamic ALC & ultra-fast 100µs squelch (uplink muting with no MS)

    - Digital filter (option)

    - Monitoring for critical network with MOR (minimum operating requirements)

# Monitoring architecture

With the secured Ewon© Talk2M© monitoring platform and a unique ID hardware, turn on your equipment and get a full monitoring solution in few minutes, with your dedicated Talk2M© account user.

# Specifications

| RF features |  |  |
| --- | --- | --- |
| Frequency bands | UL (MHz) | DL (MHz) |
|  | 136-174 | 136-174 |
| Spacing duplex | 4.45-13MHz |  |
| Bandwidth (*option: digital filter) | 1.2MHz (12.5 KHz to 1.2 MHz*) |  |
| UL POWER | +17dBm |  |
| DL POWER (*option: linearized PA) | +17dBm | +30dBm* |
| OIP3 | +48.5dBm | +56dBm |
| Rated output power per channel (DL) |  |  |
| 1 channel | +17dBm | +30dBm |
| 2 channels (60dBc) | +14dBm | +27dBm |
| 4 channels (60dBc) | +11dBm | +24dBm |
| 8 channels (60dBc) | +8dBm | +21dBm |
| 16 channels (60dBc) | +5dBm | +18dBm |
| Gains (step 0.5 dB/ precision ± 5dB) | 35-80dB | 45-90dB |
| Bandwidth ripple | ± 2dB |  |
| Noise factor | < 8dB (max gain) |  |
| Adjustment range of squelch (MS IN) | -70 to -20 dBm |  |
| Impedance | 50Ω |  |
| Delay | 4µs |  |
| Standard | ETSI compliant |  |

| Digital Filtering Module (option for DL only) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Mode | Channel of band selective |  |  |  |
| Bandwidth (channel selective) | 12.5KHz | 25KHz | 50KHz | 90KHz |
| Number of fixed filters | 18 | 12 | 9 | 4 |
| Number of dynamic filters | 10 | 6 | 5 | 2 |
| Bandwidth (band selective) |  | 200KHz to 5MHz |  |  |
| Number of fixed bands |  | 4 |  |  |
| Delay |  | 25 to 45µs |  |  |

| DAS features |  |
| --- | --- |
| Monitoring | LoRa over optical fiber |
| Monitoring frequency | 868MHz (option 433MHz) |
| Rus allowed | 16 (6xRus for 1xOMU 1U) |
| FO max. distance | 35KM |
| FO specs. | SC-APC/WDM1310-1550nm/Popt+3dBm |
| FO redundancy | Up to 2xFO per RU with the MDAS RU COUPLER |

| Monitoring features |  |
| --- | --- |
| Alarm | Minor/major alarms on native’s dry contact Minor alarm says MOR ok Major alarm says MOR not ok |
| LED (native) | Minor (RED BLINK) Major alarm (RED permanent) |
| I/O (native) | 2x dry contact / 2x optocoupled input |
| I/O (option) | 8x dry contact / 8x optocoupled input |
| PLC (option) | EWON FLEXY 202 (RFFO) / 205(RFRF) |
| Remote monitoring platform | Talk2M© |
| Cybersecurity | ISO 27001 / IEC62443 Level 1: Physical LAN commutation Level 2: Firewall Level 3: Traffic encryption (OpenVPN/SSL/TLS) Level 4: Factory/user access policy Level5: Server infrastructure monitoring Level 6: Company policy compliance |
| Distant connection (option) | 3/4G/RJ45 |
| Local protocol | OPC UA, Modbus, MQTT, SNMP, HTTP |

| Mechanical characteristics |  |  |
| --- | --- | --- |
| RF ANT connector |  | N-Type (female) |
| Dimensions (H x P x l) |  | RFRF / FORF RU 580x150x400mm (BOX) RFFO MU 133x450x483mm (3U 19”) RFRF/ FORF RU 177x450x483mm (4U 19”) |
| Weight |  | RFRF / FORF RU 18.5kg (BOX) |
| Protection | Box | IP65 |
|  | Rack | IP20 |
| Cooling system | Box | Natural cooling convection (Internal FAN are used to increase MTBF but not required for nominal operation) |
|  | Rack | Internal fans (redundant) |
| Temperature range |  | -25 / +50 °C |
| RAL |  | 9002 |
| Relative humidity range |  | ≤95% |
| MTBF |  | 100 000h |

| Electrical characteristics |  |  |  |
| --- | --- | --- | --- |
| REPEATER (MU, RU, RFRF) |  |  |  |
| Power supply | 100-240VAC / +48VDC / +24VDC |  |  |
| Internal power converter |  | Redundant (option) |  |
| Power consumption | UL ↓ / DL → | +17dBm | +30dBm |
|  | +17dBm | 45W | 75W |
| OMU |  |  |  |
| Dimensions |  | 1U / 6xFO |  |
| Power supply / consumption |  | 100-240VAC / 50W |  |

For more information: www.see-critical.com